Front Page

Objective

1. Digital Twin Simulation

Develop a validated digital twin environment of EM wave propagation during multiple partial discharges (PD) using FDTD/FIT simulators. This must incorporate experimental data from at least three distinct PD sources (e.g., corona, surface, void) and be considered complete upon achieving a shape similar to experimentally measured data by the end of semester 1.

2. Classification of PD Sources

Evaluate and compare the performance of at least three distinct feature extraction methods (e.g., wavelet analysis, envelop detection, statistical parameters) and three separation/clustering techniques (e.g., k-means, DBSCAN, two-step algorithm) using the collected/simulated EM signal dataset. The goal is to successfully classify the PD sources with a target classification accuracy of 90% or higher for the distinct PD types investigated in Goal 1.

3. Innovation and Optimization

Develop and implement a novel separation/clustering method by modifying or combining existing techniques (e.g., a hybrid clustering/classification approach). This novel method must demonstrate a quantifiable improvement over the best-performing technique from Goal 2 by achieving at least a 2% increase in classification accuracy or a 30% reduction in computational time to be considered successful.

Problem Statement

Partial Discharge (PD) is a localized electrical discharge that partially bridges the insulation between conductors, due to imperfections in the insulation. Under high voltage stress, the electric field in the defect exceeds a critical threshold, causing a discharge. Over time, this damage degrades the insulation, causing void expansion and chemical breakdown, ultimately leading to complete insulation breakdown [a].

Traditional methods of PD detection use voltage measuring devices across an impedance to detect a current spike in the device. This process is expensive and limited to devices electrically connected to the measuring instrument. A newer technology is to use Ultra-High Frequency (UHF) sensors to detect electromagnetic (EM) waves emitted from the discharges. Utilising the time difference of arrival (TDOA) of the signals at different receivers, the PD location can be determined [e]. Multiple optimization methods to localize PD from the TDOA have been developed. However, many of these methods only apply to localizing one source. In practice, there will always be multiple PD sources discharging repeatedly, overlapping, attenuating, refracting and reflecting and taking multiple paths as they travel through obstacles in a substation to reach the receiver. Traditional feature extraction methods, such as the phase resolved partial discharge (PRPD) plot, do not clearly show the differences between signals from different PD sources [s]. Signals often overlap, and higher amplitude signals can mask lower amplitude ones. Therefore, a separation algorithm is needed to classify incoming signals according to their sources for UHF radiometric detection, and to separate mixed signals successfully.

Furthermore, obtaining real world PD data for development purposes pose many challenges. First, real world data would contain interference from locally transmitted radio, as well as signal attenuation [g]. This would require knowledge on signal denoising for proper development to be done, raising the bar of entry into the field. In addition, accurately emulating PD would require a lot of resources to fabricate the PD emulator and obtain HV test equipment. Even so, emulators may not represent a wide range of possible PD types that can occur. Obtaining access to enter a substation to do testing also proves to be difficult to do, requiring permission from the relevant authorities. Therefore, a method to accurately simulate UHF signals obtained from PD sources in different environments needs to be developed, allowing for separation algorithms to be tested preliminarily before deploying them to real-world scenarios. In fact, this field has been at the forefront of PD research for many years [am].

Literature Review

The detection, classification and localization of PD have been under research since 1898 [c]. Traditionally, according to IEC 60270, PD would be measured using a coupling capacitor in parallel with the device under test (DUT) to supply charge for the current spike. Using an impedance to convert the current spike into output voltage, it is measured via a suitable coupling device (CD) and measuring instrument (MI) [b]. A high-frequency current transformer (HFCT) can also be used to obtain a voltage signal of the current spike. Non-contact methods, which have been recently popularized in research, such as ultra-high frequency (UHF) and acoustic methods are introduced in IEC 62478 [d]. There are several methods to localize a PD source from its emitted UHF waveforms. Time difference of arrival (TDOA) utilizes the difference in time between arriving signals at different receivers to estimate the distance of the PD from each of them. The exact moment of signal arrival is usually determined from the first peak or cumulative energy plot. A set of equations that represent the relationship between TDOA and PD location is solved using optimization algorithms, such as Newton-Raphson, particle swarm optimization and ant colony optimization [e]. The direction of arrival (DOA) method uses phased antenna arrays to determine which direction the PD is coming from, while the received signal strength (RSS) method uses the principle that the strength of received signal corresponds to the distance travelled by signal [f] [g].

When multiple PD sources are repeatedly discharging and emitting signals, it becomes difficult to differentiate each received waveform. Multiple methods to separate and classify signals obtained from the voltage output waveform obtained from IEC 60270/HFCT measurements have been developed. Typically, features are extracted from the signals in the time, frequency or time-frequency domain, and a clustering algorithm is then applied. The signals arriving at a sensor can either be overlapped or non-overlapped. The non-overlapped case is simpler. One example of a developed method for that case is wavelet decomposition (WD) + principal component analysis (PCA) + density-based spatial clustering of applications with noise (DBSCAN). WD passes the signal through a series of complementary high pass and low pass filters, to decompose it into detail and approximate coefficients. The signal can then be represented by scaled and shifted versions of the Mother Wavelet, a basis function defined by the user. PCA reduces the dimensionality of the sources to 3 and DBSCAN uses density to form clusters [h]. Another method is to use simple geometrical structures to form envelopes of the waveform, and create a time-frequency sparsity map [i]. Similar methods can be found in [j] to [q]. The overlapped case is more complex and requires blind source separation (BSS), a process to disentangle mixed signals [ae]. Research has shown that wavelet analysis and envelope comparison can also be used to separate sources from UHF EM waveforms [ac] [ad].

Machine learning has also been explored and applied to the field of PD classification and separation. Almehdhar & Prochazka detail an application of time-frequency analysis methods, like scatter wavelet transform (SWT) and Hilbert-Huang transform (HHT) to extract PD features, and deep learning (DL) models to classify them [r]. They achieved 100% experimental accuracy in PD separation. Carvalho et al. experimented with machine learning (ML) models (K-means, gaussian mixture model, mean-shift) to classify PD according to features such as crest factor, skewness and kurtosis, achieving 93% accuracy [s]. Further research on ML for PD classification is stated in [t] – [ab].

Most attempts at creating a digital twin of PD occurrence, represent partial discharge using a gaussian pulse of varying physical lengths, amplitudes and pulse widths, in software such as XFDTD or CST Microwave Studio. The software can numerically solve the E and H fields surrounding the PD source using the principles of finite-difference time domain (FDTD) or finite integration technique (FIT). Attempts at simulating PD are given in [af] – [ao]. A notable exception would be [ak], which does not use a gaussian pulse, but rather models the current pulse using the dielectric barrier discharge (DBD) model. Most of the simulations were done with the purpose of validating the accuracy of simulated single PD waveforms, compared to experimental measured waveforms. Only [an] and [ao] investigated the simulation of multiple PD sources for the purpose of applying separation/classification algorithms to them. He et al. implement a method to separate gaussian-pulse simulated PD signals from signal bispectra characteristics. The algorithm is then experimentally validated in a substation [ao]. More research would be needed to confirm the eligibility of testing separation algorithms on simulated PD waveforms, on whether a simulation would be capable of producing different types of PD emitted waveforms that accurately resemble experimentally collected samples, with extractable features that are unique to each different simulated PD.

Proposed Methodology

- Digital Twin Simulation Method

To achieve the first objective, a digital twin environment would be created. Two different methods of modelling a PD occurrence will be explored. The first is to use a gaussian current pulse and simulate the EM waves propagating from it:

It=Imaxe-t2/2σ2

Where Imax is maximum current amplitude, σ is the pulse width [am].

The second would be to use the dielectric barrier discharge model to compute the current pulse waveform [ak].

To simulate the propagation of EM waves from the PD source, two simulation software have been identified. The first would be CST Microwave Studio, which uses the finite integral technique (FIT), a close relative of FDTD to model EM wave propagation. It is simpler to use because of its GUI but the educational license lacks capability. The second would be an open-source software called OpenEMS, which uses FDTD and has a steeper learning curve due to its command line nature, but would have more computing power and options.

The FDTD method involves solving Maxwell’s equations in the time domain using finite-difference approximations. It discretizes space into Yee cells, with E and H components computed at the edges.

Figure _: A Yee cell [am].

Time is discretized using time steps, with E and H components updated at each time step.

- Feature Extraction Methods

Four methods to extract features from the simulated/experimental PD UHF waveform have been identified: wavelet analysis [ac], envelope detection [ad], selected bispectra [ao], and statistical analysis (skewness, kurtosis, crest factor, etc). These four would be implemented, and their performances compared. The algorithms would be implemented in Jupyter notebook for simplicity. Innovations such as applying a method not created specifically for UHF waveforms, or modifying existing methods will be experimented with.

- Clustering/Separation Methods

Several different clustering/separation methods have been identified during the literature review, such as the K-means algorithm, two-step algorithm, DBSCAN, and deep learning (DL). Algorithms will be selected for implementation and comparison based on their proven success rate and suitability. Again, innovations mentioned above would be carried out. In addition, if time permits, an attempt to create a neural network for PD classification will be made. Google Colab will be used for the construction and training.

- Experimental Validation

Planar monopole antennas and a digital oscilloscope will be used to collect EM waves. Multiple types of PD sources will be emulated using piezoelectric discharge sources. If feasible, other types of PD emulators will be obtained or constructed, such as a point plane electrode or insulation defect model, shown in the figures below.

Figure _: Insulation defect model (left) and point plane electrode (right) [ap].

References

[a] N. K. Roy, J. K. Pandey, and J. P. R. (2023). "A Review on Partial Discharge Behavior in Insulators," International Journal of Engineering, Science and Technology, vol. 15, no. 1, pp. 1-14.

[b] IEC 60270, High-voltage test techniques – Charge-based measurement of partial discharges

[c] G. C. Stone, A. Cavallini and G. Behrmann, "A Review of the History of the Development of Partial Discharge Testing," 2022 IEEE Electrical Insulation Conference (EIC), Knoxville, TN, USA, 2022, pp. 85-89, doi: 10.1109/EIC51169.2022.9833210.

[d] IEC 62478, High voltage test techniques - Measurement of partial discharges by electromagnetic and acoustic methods

[e] J. Q. Chan, S. M. K. Azam, W. J. K. Raymond, H. A. Illias, and M. Othman, “Interference-compensated optimization of UHF partial discharge localization for open environment,” Measurement, vol. 257, p. 118916, Sep. 2025, doi: 10.1016/j.measurement.2025.118916.

[f] Z. Chen, “Review of direction of arrival estimation algorithms for partial discharge localisation in transformers,” IET Science Measurement & Technology, vol. 13, no. 4, pp. 529–535, Jan. 2019, doi: 10.1049/iet-smt.2018.5297.

[g] D. W. Upton et al., “A review of techniques for RSS-Based Radiometric Partial Discharge Localization,” Sensors, vol. 21, no. 3, p. 909, Jan. 2021, doi: 10.3390/s21030909.

[h] L. Hao et al., "Discrimination of multiple PD sources using wavelet decomposition and principal component analysis," in IEEE Transactions on Dielectrics and Electrical Insulation, vol. 18, no. 5, pp. 1702-1711, October 2011, doi: 10.1109/TDEI.2011.6032842.

[i] J. C. Chan, H. Ma and T. K. Saha, "Time-frequency sparsity map on automatic partial discharge sources separation for power transformer condition assessment," in IEEE Transactions on Dielectrics and Electrical Insulation, vol. 22, no. 4, pp. 2271-2283, August 2015, doi: 10.1109/TDEI.2015.004836

[j] K. Wang, J. Li, S. Zhang, R. Liao, F. Wu, L. Yang, J. Li, S. Grzybowski, and J. Yan, “A hybrid algorithm based on s transform and affinity propagation clustering for separation of two simultaneously artificial partial discharge sources,” IEEE Trans. Dielectr. Electr. Insul., vol. 22, no. 2, pp. 1042–1060, Apr. 2015.

[k] R. D. Nimmo, G. Callender, and P. L. Lewin, “Methods for wavelet-based autonomous discrimination of multiple partial discharge sources,” IEEE Trans. Dielectr. Electr. Insul., vol. 24, no. 2, pp. 1131–1140, Apr. 2017.

[l] M.-X. Zhu, Q. Liu, J.-Y. Xue, J.-B. Deng, G.-J. Zhang, X.-J. Shao, W.-L. He, A.-X. Guo, and X.-W. Liu, “Self-adaptive separation of multiple partial discharge sources based on optimized feature extraction of cumulative energy function,” IEEE Trans. Dielectr. Electr. Insul., vol. 24, no. 1, pp. 246–258, Feb. 2017.

[m] M. Zhu, Y. Wang, D. Chang, G. Zhang, X. Shao, J. Zhan, and J. Chen, “Discrimination of three or more partial discharge sources by multi-step clustering of cumulative energy features,” IET Sci., Meas. Technol., vol. 13, no. 2, pp. 149–159, Mar. 2019.

[n] K. Firuzi, M. Vakilian, V. P. Darabad, B. T. Phung, and T. R. Blackburn, “A novel method for differentiating and clustering multiple partial discharge sources using s transform and bag of words feature,” IEEE Trans. Dielectr. Electr. Insul., vol. 24, no. 6, pp. 3694–3702, Dec. 2017.

[o] R. Albarracín-Sánchez, F. Álvarez-Gómez, C. A. Vera-Romero, and J. M. Rodríguez-Serna, “Separation of partial discharge sources measured in the high-frequency range with HFCT sensors using PRPD-teff patterns,” Sensors, vol. 20, no. 2, p. 382, Jan. 2020.

[p] G. Robles, M. Shafiq, and J. M. Martínez-Tarifa, “Multiple partial discharge source localization in power cables through power spectral separation and time-domain reflectometry,” IEEE Trans. Instrum. Meas., vol. 68, no. 12, pp. 4703–4711, Dec. 2019.

[q] Y. Li and Z. Li, “Improved density peak clustering for separation of multiple source partial discharge in large generators,” Meas. Sci. Technol., vol. 31, no. 1, p. 15013, 2020.

[r] A. Almehdhar and R. Prochazka, “Classification of multiple partial discharge sources using Time-Frequency analysis and deep learning,” Applied Sciences, vol. 15, no. 10, p. 5455, May 2025, doi: 10.3390/app15105455.

[s] I. F. Carvalho, E. G. Da Costa, L. A. M. M. Nobrega, and A. D. Da Costa Silva, “Identification of Partial Discharge Sources by Feature Extraction from a Signal Conditioning System,” Sensors, vol. 24, no. 7, p. 2226, Mar. 2024, doi: 10.3390/s24072226.

[t] Y. Fu et al., "A Partial Discharge Signal Separation Method Applicable for Various Sensors Based on Time-Frequency Feature Extraction of t-SNE," IEEE Trans. Instrum. Meas., vol. 73, pp. 3505609, 2023.

[u] Z. Zhang et al., "Partial Discharge Pattern Recognition Based on a Multifrequency F-P Sensing Array, AOK Time-Frequency Representation, and Deep Learning," IEEE Trans. Dielectr. Electr. Insul., vol. 29, no. 4, pp. 1701–1710, 2022.

[v] A. Rauscher, J. Kaiser, M. Devaraju, and C. Endisch, "Deep learning and data augmentation for partial discharge detection in electrical machines," Eng. Appl. Artif. Intell., vol. 133, p. 108074, 2024.

[w] B. Chen, Y. Hu, L. Wu, W. Xu, and H. Sun, "Partial Discharge Identification Based on Unsupervised Representation Learning Under Repetitive Impulse Excitation With Ultra-Fast Slew-Rate," IEEE Trans. Power Deliv., vol. 39, no. 2, pp. 801–810, 2024.

[x] A. Li et al., "Pattern Recognition of Partial Discharge in High-Voltage Cables Using TFMT Model," IEEE Trans. Power Deliv., vol. 39, no. 1, pp. 336–337, 2024.

[y] R. Yang et al., "Detection of Partial Discharge Acoustic Signals in Converter Valve Halls Using Multivariate Time-Frequency Slices," IEEE Sens. J., Early Access.

[z] J. Zheng, Z. Chen, Q. Wang, H. Qiang, and W. Xu, "GIS Partial Discharge Pattern Recognition Based on Time-Frequency Features and Improved Convolutional Neural Network," Energies, vol. 15, no. 19, p. 7372, 2022.

[aa] B. Chen, Y. Hu, and L. Wu, "Deep Learning-Based Multi-Source Partial Discharge Pattern Recognition Integrated with Auxiliary Prior Localization Information," IEEE Trans. Dielectr. Electr. Insul., In Press, pp. 1–9.

[ab] R. Sahoo and S. Karmakar, "Comparative analysis of machine learning and deep learning techniques on classification of artificially created partial discharge signal," Measurement, vol. 235, p. 114947, 2024.

[ac] L. Yang and M. D. Judd, “Recognising multiple partial discharge sources in power transformers by wavelet analysis of UHF signals,” IEE Proceedings - Science Measurement and Technology, vol. 150, no. 3, pp. 119–127, Apr. 2003, doi: 10.1049/ip-smt:20030201.

[ad] T. Pinpart and M. D. Judd, “Differentiating between partial discharge sources using envelope comparison of ultra-high-frequency signals,” IET Science Measurement & Technology, vol. 4, no. 5, pp. 256–267, Aug. 2010, doi: 10.1049/iet-smt.2009.0064.

[ae] J. Tang, W. Li, and Y. Liu, “Blind source separation of mixed PD signals produced by multiple insulation defects in GIS,” IEEE Trans. Power Del., vol. 25, no. 1, pp. 170–176, Jan. 2010.

[af] M. Zanjani, A. Akbari, H. R. Mirzaei, N. Shirdel, E. Gockenbach and H. Borsi, "Investigating partial discharge UHF electromagnetic waves propagation in transformers using FDTD technique and 3D simulation," 2012 IEEE International Conference on Condition Monitoring and Diagnosis, Bali, Indonesia, 2012, pp. 497-500, doi: 10.1109/CMD.2012.6416187.

[ag] A. Jaber et al., "Validation of partial discharge emulator simulations using free-space radiometric measurements," 2016 International Conference for Students on Applied Engineering (ICSAE), Newcastle Upon Tyne, UK, 2016, pp. 475-478, doi: 10.1109/ICSAE.2016.7810238.

[ah] S. Ohtsuka, T. Teshima, S. Matsumoto and M. Hikita, "Relationship between PD-induced electromagnetic wave measured with UHF method and charge quantity obtained by PD current waveform in model GIS," 2006 IEEE Conference on Electrical Insulation and Dielectric Phenomena, Kansas City, MO, USA, 2006, pp. 615-618, doi: 10.1109/CEIDP.2006.312007.

[ai] Toshihiro Hoshino, Shiro Maruyama and Takaaki Sakakibara, "Simulation of propagating electromagnetic wave due to partial discharge in GIS using FDTD," 2008 International Conference on Condition Monitoring and Diagnosis, Beijing, China, 2008, pp. 1258-1261, doi: 10.1109/CMD.2008.4580205.

[aj] T. Umemoto and S. Tenbohlen, “Novel Simulation Technique of Electromagnetic Wave Propagation in the Ultra High Frequency Range within Power Transformers,” Sensors, vol. 18, no. 12, p. 4236, Dec. 2018, doi: 10.3390/s18124236.

[ak] Q. Zheng, M. Wang, L. Luo, G. Sheng and X. Jiang, "PD Signal Detection Model Based on Multiscale Simulation of EM Generation and Transmission," in IEEE Transactions on Instrumentation and Measurement, vol. 72, pp. 1-10, 2023, Art no. 8002910, doi: 10.1109/TIM.2023.3272391.

[al] Y. Tian and M. Kawada, “Estimation of DOAs of EM Waves Emitted from Multiple Partial Discharge Sources in Free Space by Using Wideband Signal Subspace Methods,” Journal of International Council on Electrical Engineering, vol. 1, no. 4, pp. 384–389, Oct. 2011, doi: 10.5370/jicee.2011.1.4.384.

[am] A. Loubani, N. Harid, H. Griffiths, and B. Barkat, “Simulation of partial discharge induced EM waves using FDTD Method—A Parametric Study,” Energies, vol. 12, no. 17, p. 3364, Sep. 2019, doi: 10.3390/en12173364.

[an] Y. Yun, G. Xu, W. Zhang, X. Li, and L. Chen, “Simulation analysis on partial discharge signals produced by multiple insulation defects in GIS,” E3S Web of Conferences, vol. 165, p. 06040, Jan. 2020, doi: 10.1051/e3sconf/202016506040.

[ao] Z. He, H. Hou, Y. Zang, G. Sheng, and X. Jiang, “A method for separating multisource partial discharges in a substation based on selected bispectra of UHF signals,” Applied Sciences, vol. 10, no. 11, p. 3751, May 2020, doi: 10.3390/app10113751.

[ap] Y. Li, J. Han, Y. Du and H. Jin, "Time–Frequency Maps for Multiple Partial Discharge Sources Separation in Cable Terminations," in IEEE Transactions on Power Delivery, vol. 38, no. 3, pp. 2228-2231, June 2023, doi: 10.1109/TPWRD.2023.3256127.

  - HFCT – high frequency current transformer at grounded end of circuit

  - DOA

    - Uses a phased array of antennas and the theory of spatial spectrum estimation to obtain direction of arrival. Multiple phased arrays at different locations can be used to pinpoint the exact location of PD.

  - RSS based localisation

    - Measure the power of electromagnetic emission at multiple sensor nodes. Received power varies with distance travelled by

    - R=R0-10nlog10d, R0 is transmitted power, n is path loss index, d is distance travelled

    - It does not require precise synchronization between sensors, making it cheaper and simpler. However, multipath propagation and signal attenuation make things complex, requiring calibration before detection. Typical methods to implement are to use envelope detection and transistor reset integrators (TRI)

- Digital Twin Simulation Method: FDTD, using gaussian pulse Simulation of Partial Discharge Induced EM Waves Using FDTD Method—A Parametric Study or Dielectric barrier discharge model https://ieeexplore.ieee.org/document/10114630

  - It=Imaxe-t2/2σ2, Imax is maximum current amplitude, σ is the pulse width.

  - (If possible) DBD: modelling the micro discharge process within voids, obtaining a theoretical current pulse which can be used as the source of EM waves in PD simulations.

  - FDTD method solves Maxwell’s equations in the time domain using finite-difference approximations. It discretizes time and space.

  - Space is discretized in the form of a Yee cell, with E and H components computed at certain locations across the cell

  - Time is discretized in the form of time steps, with E and H fields of each Yee cell computed for each time step. Several simulation software utilise FDTD principle, or its counterpart, FIT, such as OpenEMS, MEEP and CST Microwave Studio.

- Feature Extraction Method

  - Wavelet analysis (PDF) Recognising multiple partial discharge sources in power transformers by wavelet analysis of UHF signals

  - Envelope detection (PDF) Differentiating between partial discharge sources using envelope comparison of ultra-high-frequency signals

  - Crest Factor/Skewness/Kurtosis https://www.mdpi.com/1424-8220/24/7/2226

- Clustering

  - K-means algorithm

  - Two step algorithm

  - https://ieeexplore.ieee.org/document/9797234

- Measurement Method

  - Planar monopole antennas, piezoelectric discharge source, oscilloscope

  - If possible: construct own PD emulators using:

    - Point Plane Electrode Time–Frequency Maps for Multiple Partial Discharge Sources Separation in Cable Terminations | IEEE Journals & Magazine | IEEE Xplore

    - Insulation defect